COMMUNITY PARTNERED INCREMENTAL HOUSING: A SOLUTION FOR THE ISSUE OF LOW INCOME HOUSING IN LAHORE, PAKISTAN

Prof. Dr. S. Shabih-ul-Hassan Zaidi

shabih52@gmail.com

Abstract

Pakistan has been facing rapid urbanization since its independence from British rule in 1947. As a consequence, the large cities in Pakistan such as Lahore, are facing an acute shortage of housing, particularly for the low income group.  In Lahore, with a current population of nine million and a growth rate of 3.5 percent, approximately 300,000 people are added every year. Out of these around 80 percent are forced to find shelter in slums and squatter settlements, locally known as Katchi Abadis. The planned housing stock developed by the Lahore Development Authority and the private sector developers caters for the high and middle income only. The main causes of this situation are the low level of affordability of the low income people and the poor response of the government planning and development agencies to address the situation properly. It means that the planners and administrators have failed to provide benefits of town planning to the poor masses. However, one of the best planning practices also exists in Pakistan known as Khuda Ki Basti (KKB) in Hyderabad, which uses the concept of community partnered incremental development and aided self-help. The KKB model has been replicated at four places including Lahore in Pakistan. It has demonstrated a high level of success in reaching to the low income group. This paper reviews this model of KKB in Lahore and suggests some modifications in it for replication in all the cities of Pakistan to help the low income people in procuring planned housing stock for themselves. The recommendations include sustainable development of communities with income generation programmes using micro-credit schemes. Moreover, it is suggested that the KKB housing projects should not be developed as isolated housing schemes. Instead, they should be made a part of the large integrated housing schemes using the concept of cross-subsidization for the low income blocks of the scheme. The success of the community partnered incremental housing programme can play a major role in the development of planned housing stock for low income masses in Lahore, Pakistan.

RAPID URBANIZATION AND THE CREATION OF SLUMS IN THE CITIES OF PAKISTAN

Urbanization is a universal phenomenon and almost all countries have gone through this process. It is a macro level parametric phenomenon. However, the developing countries such as Pakistan are facing a trend of rapid urbanization in which about 4 percent increase in the urban proportion of population is noticed in just one decade. The current (2009) population of Pakistan is estimated to be about 170 million with a growth rate 2.61 percent per annum (Census of Pakistan, 1998).  At the time of independence in 1947, the urban proportion of population in Pakistan was around 15 percent. This proportion has risen to about 40 percent in 2009 and it is expected that by 2030 more than 50 percent of the population of Pakistan will be living in urban areas. The social scientists believe that this rapidity of urbanization may be slowed down when the majority of population starts living in the urban areas. There are three factors contributing towards the rapid urbanization of Pakistan. Firstly, it is the natural growth of population caused by high birth rate which could not be curtailed despite the efforts of the Family Planning or Population Welfare Department. This factor contributes 70 percent of the total magnitude of urbanization. Secondly, the shifting of people from rural areas to the urban areas due to unemployment and lack of public facilities and utility services in the rural areas, which causes 22 percent of the total magnitude of urbanization. The third factor is the reclassification of rural areas into urban areas due to increase in the population of villages beyond the 5000 figure and the expansion of large cities engulfing the surrounding villages into urban areas. This factor is responsible for 8 percent of the total urbanization in the country.  In Pakistan the number of urban areas (towns and cities) has increased from 337 in 1961 to 538 in 1998 [1]. Similarly, in Punjab province there were 139 urban areas in 1961 which increased to 245 in 1998 (Fig. 1). This is mainly due to uncontrolled natural growth of population  of large villages which have become urban areas with the passage of time.

Fig. 1.  Numeric Growth of Towns & Cities in Punjab and Pakistan

Source: Census of Pakistan, 1998.

One of the greatest issues created by rapid urbanization in Pakistan is the housing deficit in urban areas, particularly for low income people. According to the National Housing Policy of 2001, a backlog of 4.3 million houses existed in the country. This backlog might have crossed the 5 million figure by now. Obviously, most part of this housing backlog relates to the low income housing. Annual additional housing requirement is 570,000 units while the annual production is 300,000 units which means a recurring backlog of 270,000 units. This situation gives rise to the growth of slums and squatter settlements in the urban areas of Pakistan.

The low income housing situation in Pakistan has worsened with the rapid increase in its urban population. Today, the exorbitant land values and a tremendous rise in the cost of building materials have made it almost impossible for a low income person to procure a decent shelter in a planned housing scheme. The situation has resulted in the creation of slums and squatter settlements (locally known as Katchi Abadis) in almost all cities and towns of Pakistan. The existing slums and Katchi Abadis are expanding and rising in density. The unhygienic conditions prevail in all of these slums and Katchi Abadis because of the lack of basic utility services such as water supply and sewerage system. These settlements are the manifestations of the increasing poverty in Pakistan.

THE MAIN ISSUES OF LOW INCOME HOUSING IN PAKISTAN

The following are the main issues of Low Income Housing in Pakistan [5]:

1. There is a big gap between the cost of decent, authorized and planned housing and the affordability of the low income people.

2. The self-help potential (clearly demonstrated in the construction of squatter housing) of low income people has not been utilized.

3. Poor people are unable to obtain proper housing because of their low incomes. But poverty eradication through income generation programs was never made a part of the low income housing programs.

4. In the past, the government did not take any concrete steps of enabling, controlling and facilitating low income housing. As a result the gap between demand and supply of low income housing could not be closed. Similarly, no research was sanctioned for finding out solutions for resolving the issue of low income housing.

Various type of housing programmes were introduced in Pakistan to deal with the above issues, but most of them failed to resolve the problem of low income housing.

FAILURE OF PROGRAMS AIMING AT CONSTRUCTION OF NEW HOUSING FOR LOW INCOME PEOPLE

1. The Public Housing Programs such as Sodiwal Quarters, Riwaz Garden Flats of Lahore etc. failed  due to their insufficiency in number (as against the huge demand) and lack of understanding of the requirements of the poor people.

2. The Core housing program of Lahore Township and Green Town schemes failed due to their poor construction and inadequate design. Later they were occupied by middle income people and re-constructed with change in design.

3. The Sites and Services schemes which are currently in vogue, have not served the poor people. The small plots (5 Marla or less) reserved in public schemes have been occupied by higher income groups and are now out of reach of the poor people. The private schemes do not cater for the low income people since the developers are interested in profit making.

4. The mosaic of small private schemes results in the lack of higher order physical and social infrastructure e.g. Trunk Sewers, High Schools & Colleges and Hospitals for large populations.

5. Self help housing potential of low-income people is not being exploited in the construction of new housing. Thus the S&S Schemes become unaffordable for poor people and they remain dissatisfied even with highly subsidized housing and do not participate in the maintenance of the schemes.

6. No effort is made to introduce income generation programs with new housing schemes to eradicate poverty.

Despite the failure of various programmes initiated by government for construction of low income housing, one of the best practices also exists in Pakistan, known as Khuda-Ki-Basti (KKB) project.

KHUDA-KI-BASTI (KKB) PROJECT, HYDERABAD, PAKISTAN

The Khuda-Ki-Basti (KKB) project was initiated by the Hyderabad Development Authority (HDA), Hyderabd, Pakistan by a Planner Azharuddin Khan and Mr. Tasneem Siddiqui, the then Director General of   Hyderabad Development Authority (HDA). This was a low-income sites and services scheme introduced in the low-income blocks of Gulshan-e-Shahbaz, Hyderabad. It was developed as an answer to various problems of the common public sites and services  schemes launched by various development authorities in Pakistan. It was noticed that the existing sites and services program had failed to house the majority of low-income people who need a house. Thus the low-income people were marginalized from the public housing schemes and were forced to live in the slums and Katchi Abadies. The KKB project provided a departure from the existing system of provision of housing to the urban poor. Firstly, it changed the sequence of development of housing from the provision of completed houses in a planned and serviced housing scheme to that of provision of planned (surveyed) plots to the selected urban poor and then development of all the utility services, roads and super structure of the houses by the allottees on self-help basis. Secondly, it introduced the idea of incremental development of infrastructure services and the construction of houses as and when the allottees can afford them on self-help basis. Community organization was at the back of this approach. The KKB approach proved to be successful in the provision of planned and decent housing for the urban poor at affordable cost. The approach has been replicated at 4 different places up till now, including the one at Kala Shah Kaku (KSK) near Lahore.

The following were the main considerations in the Khuda-Ki-Basti project:

- The KKB used the Incremental Development Approach to combat the issue of low level of housing affordability of the low-income people.

- The KKB adopted the new innovative allotment criteria for allotment of plots to all the needy low-income people without balloting. Thus provisional allotment of a surveyed plot is made on need base criteria. Those who do not want to start living in the project area immediately or those who do not want to participate in a self-help and incremental development housing project are not allotted any plot.

- The project utilized the low-income people’s self-help potential in the development of infrastructure as well as construction of houses.

- The KKB was an aided self-help program for the development of new and planned low-income housing stock by involving technical staff of the HDA.

- The KKB helped in bridging the time lag between development of a sites and services scheme and construction of houses in the scheme which has become one of the greatest issues of the traditional sites and services schemes.

- The KKB achieved community organization and empowerment of the poor people through a self-help housing project.

RECOMMENDATIONS FOR LOW INCOME HOUSING IN PAKISTAN

The following recommendations are made in the light of the review of the traditional low income housing programs and the best practice of KKB available in Pakistan:

1. Development of KKB Type Housing Schemes as a part of Master Plans:

The Government of Pakistan, through proper Town Planning legislation, should ensure the preparation and implementation of Master Plans for all the cities and urban areas of the country. The local planning authorities should develop a number of local district plans continuously for larger areas as part of the Master Planning process. These local plans should be developed by government and should contain large shopping centers, major roads, trunk infrastructure and sites for higher level public facilities such as hospitals and colleges. The Low Income Housing programs should be launched in every city as a part of the Master Plan which should provide policy guidelines regarding the housing supply, space standards and the location of housing schemes to be developed by private and public sectors. The low income housing should be developed in the form of Khuda-Ki-Basti schemes as a part of the Master Plan. The homeless people living in urban areas should be accommodated in the Khuda-Ki-Basti type of incremental development housing schemes. These schemes provide surveyed plots in a planned housing scheme. The houses as well as physical and social infrastructure are developed incrementally by the allottees on self help basis. The incremental housing development program should be targeted to the poor people living in rental houses in the slums. Such schemes can be initiated by the local development authorities or TMAs as well as by the local NGOs such as Saiban.

2. Cross Subsidy Housing Schemes:

The existing private and public sites and services programs should be modified by incorporating the concept of KKB schemes so as to accommodate low income people adequately. It is recommended that all public housing schemes should reserve 75 percent plots of 3, 4 and 5 Marlas (average size of 4 Marlas) for low income people, 20 percent plots having an average size of 10 Marlas for middle income people, and 5 percent plots of 1 Kanal each for the high income people. This type of housing scheme will have an equal distribution of land under developed plots between low income group and the middle plus high income groups. The middle and high income allottees of the plots should be charged  50 percent higher price than the average cost of developed plots while the low income people will pay 50 percent less than the average cost of developed land.  Thus, in such a housing scheme, if for example, average cost of developed plots is 1 million rupees per Kanal, the middle and high income plots should be sold at 1.5 million rupees per Kanal, while the low income plots should be sold at 0.5 million rupees per Kanal. Thus low income people will get a plot of an average size of 4 marlas at a very affordable price of Rs.100,000 in the low income blocks of the scheme. These plots (75% of the total plots) should be allotted to the low income people using the allotment criteria of the KKB and the low income people should be encouraged to build their houses incrementally on self-help basis. A community organization should be initiated by local planning agency or NGO in each block of these low income plots and this organization should develop the area in the form of small communities working together for procuring a decent and planned housing for themselves.

The allotment criteria in all the above schemes should be such that only those low income people get the provisional allotment of plots who are homeless and want to live there immediately. More over, the standard of streets in middle and high income blocks should be 40 to 60 feet wide black top roads while in low income blocks of housing the width of streets should be 30 feet with brick pavement to discourage occupation of subsidized low income plots by higher income groups. More over, the low income allottees of these subsidized plots will not be allowed to sell these plots for at least 5 years after the allotment. If they are press hard to leave the scheme, they will be given an option of selling these plots or houses to the housing authority after deduction of 10 percent management charges. Thus cross-subsidized low income plots will become affordable for the urban poor while the high and middle income people will be discouraged to buy these plots from the low income people.

3. Labour Colonies Near Industrial Areas and Posh Housing Schemes:

The government with assistance from the industrialists should develop labour colonies near the industrial estates. The government should enact laws requiring the owners of large industries (employing 200 or more workers) to build housing for at least 50 percent of their employees near their industries (with sufficient safeguard from environmental hazards). Similarly, small walkup apartment buildings should be developed for low income government employees in the posh Government colonies such as Government Officers Residences (GOR) Colony and  Mayo Gardens in Lahore by demolishing 25 percent of spacious houses in these schemes. This type of low income housing schemes will help to reduce average trip lengths to job places, schooling and shopping purposes which will ultimately help to conserve energy and reduce pollution in the cities.

Additionally, the Government employees should be provided housing at the time of retirement as per existing program of armed forces of Pakistan. All Master Plans should incorporate this housing program. Small residential apartments should be built in the form of four storey walkup buildings for low income employees within the premises of the government organizations where ever possible.

4. Owner Occupied and Rental Accommodation for the Urban Poor by the Provincial and Local Governments:

The provincial governments should identify vacant state lands in and around the cities and build multistory flats such as Pakistan Housing Authority (PHA) flats. Fifty percent of these flats may be allotted to low income people on installment basis, while the rest of flats may be provided to low income people on rental basis at subsidized rents. This type of housing may also be developed by the local councils (TMAs) on the pattern of the Council Housing in the United Kingdom and other developed countries. This program will also help to improve the assets and income of the Local Governments. The owner occupied flats should be provided on 50 years lease basis to the really low income people on a subsidized rate. These flats should be non-transferable and in case somebody wants to leave, he should sell it back to the Government at reserve price. The rental flats should be allotted on need basis and their merit should be carefully determined using a Performa with a number system determining the urgency of requirement by the applicants.

5.  Land Pooling and Readjustment Schemes through Private-Public Partnership:

In the urban areas where small land holdings exist, land pooling and readjustment schemes should be developed through private-public partnership. The local development authorities or TMAs should initiate a land pooling program by negotiating with the the private land owners. The public authority should accumulate the small pieces of land and plan and develop it on a unified basis. After the development of the housing scheme, the private land owners can obtain the developed plots according to their share of land in the scheme. However, development charges should be paid in full by the participant land owners or a portion of their developed plots can be retained by the public development authority for accommodating the low income people. The land owners may also contribute in advance towards the cost of development of housing scheme. Through this program the public authority can ensure planned development of all land holdings and retain public land in the form of roads, public buildings area, parks and open spaces at no cost to the public exchequer.

6. Short Term Loans for Purchase of Plots:

Short term (3 to 5 years) soft loans may be arranged by Government (House Building Finance Corporation) for low income people to procure plots in the new private and public sites and services housing schemes. Later the Government may facilitate the construction of houses on these plots through an aided self-help program such as Building Together Project of Bangkok, Thailand and the Khuda-Ki-Basti Project of Hyderabad, Pakistan.

7. Poverty Alleviation and Income Generation Progarm:

Poverty Alleviation and Income Generation programs such as provision of job opportunities in the form of shop-houses, open markets and micro credit schemes should be introduced in all the above schemes. In case of multistory flats, ground floor of four storey apartments should be developed as shopping centre in which shops should be allotted to low income residents.

REFERENCES

1. Census of Pakistan, 1998.

2. Zaidi, S. H. and Mayo, Shaker, ‘New Urbanization Trends in Punjab, Pakistan and Their

Effect on The Lahore Metropolis’, Paper presented in the 10th Asian Urbanization Conference, held at the University of Hong Kong, 16-19 August, 2009.

3.  ibid.

4. UN-HABITAT, Basic Facts on Urbanization, Nairobi: United Nations Center for Human Settlements (Habitat), 1999.

5. Zaidi, S. H., ‘Mitigating Houses Issues in Pakistan’, in Cities – Engines of Growth, Edited by Nadeem  Ul  Haq and Durr-e-Nayab, Pakistan Institute of Development Economics, Islamabad, 2007.

6.  ibid.

7.  Bajwa, I. and Zaidi, S. H., ‘Why Master Plans Fail to be Implemented in Pakistan’, Proceedings of the Second Seminar on Urban and Regional Planning, Department of Architecture and Planning, NED University of Engineering and Technology, Karachi, 2005.